| Voltage rating |  | AC220V 50/60Hz |
| --- | --- | --- |
| Voltage limit |  | AC 180V~250V |
| Hysteresis |  | ≤2sec/day(25 ℃ ) |
| On/off operation |  | 45 On & 45 Off，22 times pulse |
| Power consumption |  | 5VA(max) |
| Dispaly |  | LCD |
| Minimum interval |  | 1 Minute(pulse/cycle 1 s) |
| Pulse |  | 1 Sec-59min 59 sec |
| Count down |  | 1 Sec-99 min 59 sec |
| Width |  | 2 Modules(36mm) |
| Installation Type |  | Din Rail |
| Power reserve |  | 3 Years (Lithium battery) |
| Switching contact |  | 1 Change over switch |
| Switching Contact at 250VAC, cosφ=1 |  | 16A |
| Switching capacity at 250VAC, cosφ=0.6 |  | 10A |
| Incandescent/HALogen lamp load |  | 2300W |
| Energy saving lamps |  | 22x7W, 18X11W, 16X15WM 16X20W, 14X23W |
| Service life | Mechancally | 107 |
|  | electrically | 105 |
| Ambient humidity |  | 35~85%rh |
| Ambient temperature |  | -10~+40 ℃ |
| Weight |  | approx150g |
| Certificate |  | CE EMC, CE LVD, Rohs |